DROWSINESS DETECTION

Drowsiness_detector.py

import cv2

import dlib

import numpy as np

from scipy.spatial import distance as dist

import time

import platform

import requests   # <<< added for ESP8266 communication

# ================== CONFIG ==================

EYE_AR_THRESH = 0.23

EYE_AR_CONSEC_FRAMES = 20

ESP_IP = "192.168.4.1"   # <<< change if your ESP IP is different

SEND_STOP_ON_ALERT = True

USE_BEEP = True if platform.system() == "Windows" else False

if USE_BEEP:

import winsound

# ============================================

# ========== ROBOT CONTROL ==========

def stop_robot():

if not SEND_STOP_ON_ALERT:

return

try:

requests.get(f"http://{ESP_IP}/STOP", timeout=1)

print("[INFO] STOP sent to robot")

except Exception as e:

print("[WARN] Could not contact robot:", e)

# ========== HELPER FUNCTIONS ==========

def eye_aspect_ratio(eye):

A = dist.euclidean(eye[1], eye[5])

B = dist.euclidean(eye[2], eye[4])

C = dist.euclidean(eye[0], eye[3])

ear = (A + B) / (2.0 * C)

return ear

def shape_to_np(shape, dtype="int"):

coords = np.zeros((68, 2), dtype=dtype)

for i in range(0, 68):

coords[i] = (shape.part(i).x, shape.part(i).y)

return coords

RIGHT_EYE_START, RIGHT_EYE_END = 36, 42

LEFT_EYE_START, LEFT_EYE_END = 42, 48

# ========== LOAD MODELS ==========

print("[INFO] Loading facial landmark predictor...")

detector = dlib.get_frontal_face_detector()

predictor = dlib.shape_predictor("shape_predictor_68_face_landmarks.dat")

print("[INFO] Starting video stream...")

cap = cv2.VideoCapture(0)

time.sleep(1.0)

COUNTER = 0

ALARM_ON = False

# ========== MAIN LOOP ==========

while True:

ret, frame = cap.read()

if not ret:

print("[ERROR] Camera frame missing")

break

frame = cv2.resize(frame, (640, 480))

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

rects = detector(gray, 0)

for rect in rects:

shape = predictor(gray, rect)

shape = shape_to_np(shape)

leftEye = shape[LEFT_EYE_START:LEFT_EYE_END]

rightEye = shape[RIGHT_EYE_START:RIGHT_EYE_END]

leftEAR = eye_aspect_ratio(leftEye)

rightEAR = eye_aspect_ratio(rightEye)

ear = (leftEAR + rightEAR) / 2.0

leftHull = cv2.convexHull(leftEye)

rightHull = cv2.convexHull(rightEye)

cv2.drawContours(frame, [leftHull], -1, (0, 255, 0), 1)

cv2.drawContours(frame, [rightHull], -1, (0, 255, 0), 1)

# ====== Drowsiness Logic ======

if ear < EYE_AR_THRESH:

COUNTER += 1

if COUNTER >= EYE_AR_CONSEC_FRAMES:

if not ALARM_ON:

ALARM_ON = True

print("[ALERT] Drowsiness detected!")

stop_robot()   # <<< ROBOT STOP HERE

if USE_BEEP:

winsound.Beep(2500, 1000)

cv2.putText(frame, "DROWSY!", (10, 60),

cv2.FONT_HERSHEY_SIMPLEX, 1.2,

(0, 0, 255), 3)

else:

COUNTER = 0

ALARM_ON = False

cv2.putText(frame, f"EAR: {ear:.2f}", (10, 30),

cv2.FONT_HERSHEY_SIMPLEX, 0.7,

(255, 255, 0), 2)

cv2.imshow("Driver Drowsiness Detection", frame)

key = cv2.waitKey(1) & 0xFF

if key == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

requirements.txt

opencv-python

dlib

numpy

pillow

drowainess_yawn_logger.py

import cv2

import dlib

import numpy as np

from scipy.spatial import distance as dist

import time

import platform

import csv

from datetime import datetime

# ========== CONFIGURABLE PARAMETERS ==========

# Eye Aspect Ratio

EYE_AR_THRESH = 0.23            # EAR below this → eyes considered closed

EYE_AR_CONSEC_FRAMES = 20       # frames required to trigger drowsiness alarm

# Mouth Aspect Ratio for yawning

MOUTH_AR_THRESH = 0.60          # MAR above this → mouth considered open (yawn)

MOUTH_AR_CONSEC_FRAMES = 15     # frames required to trigger yawning detection

# CSV log filename

LOG_FILENAME = "events_log.csv"

# Optional: Beep support (Windows only)

USE_BEEP = True if platform.system() == "Windows" else False

if USE_BEEP:

import winsound

# ========== HELPER FUNCTIONS ==========

def eye_aspect_ratio(eye):

# eye: 6 (x, y) points

A = dist.euclidean(eye[1], eye[5])

B = dist.euclidean(eye[2], eye[4])

C = dist.euclidean(eye[0], eye[3])

ear = (A + B) / (2.0 * C)

return ear

def mouth_aspect_ratio(mouth):

# Using outer lip: 48–60 (12 points)

# Points: 49, 53, 51, 57, 50, 58, 52, 56, 48, 54

# A and B are vertical distances, C is horizontal

A = dist.euclidean(mouth[2], mouth[10])  # 51, 59

B = dist.euclidean(mouth[4], mouth[8])   # 53, 57

C = dist.euclidean(mouth[0], mouth[6])   # 49, 55

mar = (A + B) / (2.0 * C)

return mar

def shape_to_np(shape, dtype="int"):

coords = np.zeros((68, 2), dtype=dtype)

for i in range(0, 68):

coords[i] = (shape.part(i).x, shape.part(i).y)

return coords

def log_event(event_type, ear_value, mar_value):

"""

Log event to CSV: timestamp, event, ear, mar

"""

now = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

with open(LOG_FILENAME, mode="a", newline="") as f:

writer = csv.writer(f)

writer.writerow([now, event_type, f"{ear_value:.3f}", f"{mar_value:.3f}"])

print(f"[LOG] {event_type} at {now} (EAR={ear_value:.3f}, MAR={mar_value:.3f})")

# ========== LANDMARK INDICES ==========

# Eyes

RIGHT_EYE_START, RIGHT_EYE_END = 36, 42   # 36–41

LEFT_EYE_START, LEFT_EYE_END = 42, 48    # 42–47

# Mouth (outer): 48–60

MOUTH_START, MOUTH_END = 48, 61

# ========== CHECK/CREATE CSV HEADER ==========

try:

# If file doesn't exist, this will throw

with open(LOG_FILENAME, mode="x", newline="") as f:

writer = csv.writer(f)

writer.writerow(["timestamp", "event", "ear", "mar"])

except FileExistsError:

# File already exists → do nothing

pass

# ========== INITIALIZE DLIB MODELS ==========

print("[INFO] Loading facial landmark predictor...")

detector = dlib.get_frontal_face_detector()

predictor = dlib.shape_predictor("shape_predictor_68_face_landmarks.dat")

# ========== START VIDEO CAPTURE ==========

print("[INFO] Starting video stream...")

cap = cv2.VideoCapture(0)

time.sleep(1.0)

# Counters and flags

eye_counter = 0

mouth_counter = 0

drowsy_alarm_on = False

yawn_alarm_on = False

while True:

ret, frame = cap.read()

if not ret:

print("[ERROR] Failed to grab frame from camera.")

break

frame = cv2.resize(frame, (640, 480))

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

rects = detector(gray, 0)

# Default values if no face is found

ear = 0.0

mar = 0.0

drowsy_detected = False

yawn_detected = False

for rect in rects:

shape = predictor(gray, rect)

shape = shape_to_np(shape)

# Eyes

leftEye = shape[LEFT_EYE_START:LEFT_EYE_END]

rightEye = shape[RIGHT_EYE_START:RIGHT_EYE_END]

leftEAR = eye_aspect_ratio(leftEye)

rightEAR = eye_aspect_ratio(rightEye)

ear = (leftEAR + rightEAR) / 2.0

# Draw eye contours

leftEyeHull = cv2.convexHull(leftEye)

rightEyeHull = cv2.convexHull(rightEye)

cv2.drawContours(frame, [leftEyeHull], -1, (0, 255, 0), 1)

cv2.drawContours(frame, [rightEyeHull], -1, (0, 255, 0), 1)

# Mouth

mouth = shape[MOUTH_START:MOUTH_END]

mar = mouth_aspect_ratio(mouth)

# Draw mouth contour

mouthHull = cv2.convexHull(mouth)

cv2.drawContours(frame, [mouthHull], -1, (255, 0, 0), 1)

# --- DROWSINESS BY EYE CLOSURE ---

if ear < EYE_AR_THRESH:

eye_counter += 1

else:

eye_counter = 0

drowsy_alarm_on = False

if eye_counter >= EYE_AR_CONSEC_FRAMES:

drowsy_detected = True

if not drowsy_alarm_on:

drowsy_alarm_on = True

if USE_BEEP:

winsound.Beep(2500, 800)  # frequency, duration

# --- YAWNING BY MOUTH OPENING ---

if mar > MOUTH_AR_THRESH:

mouth_counter += 1

else:

mouth_counter = 0

yawn_alarm_on = False

if mouth_counter >= MOUTH_AR_CONSEC_FRAMES:

yawn_detected = True

if not yawn_alarm_on:

yawn_alarm_on = True

if USE_BEEP:

winsound.Beep(1800, 600)

# --- VISUAL TEXT OVERLAYS ---

if drowsy_detected:

cv2.putText(frame, "DROWSY!", (10, 80),

cv2.FONT_HERSHEY_SIMPLEX, 1.0, (0, 0, 255), 3)

if yawn_detected:

cv2.putText(frame, "YAWNING!", (10, 120),

cv2.FONT_HERSHEY_SIMPLEX, 1.0, (255, 0, 255), 3)

# Display EAR and MAR

cv2.putText(frame, f"EAR: {ear:.2f}", (10, 30),

cv2.FONT_HERSHEY_SIMPLEX, 0.7, (255, 255, 0), 2)

cv2.putText(frame, f"MAR: {mar:.2f}", (10, 55),

cv2.FONT_HERSHEY_SIMPLEX, 0.7, (255, 255, 0), 2)

# --- LOG EVENTS (ONCE PER EPISODE) ---

if drowsy_detected or yawn_detected:

if drowsy_detected and yawn_detected:

event_type = "BOTH"

elif drowsy_detected:

event_type = "DROWSY"

else:

event_type = "YAWN"

# Log once when each alarm triggers

if (drowsy_detected and drowsy_alarm_on and eye_counter == EYE_AR_CONSEC_FRAMES) or \

(yawn_detected and yawn_alarm_on and mouth_counter == MOUTH_AR_CONSEC_FRAMES) or \

(event_type == "BOTH" and (

eye_counter == EYE_AR_CONSEC_FRAMES or mouth_counter == MOUTH_AR_CONSEC_FRAMES

)):

log_event(event_type, ear, mar)

cv2.imshow("Driver Drowsiness & Yawn Detection", frame)

key = cv2.waitKey(1) & 0xFF

if key == ord('q'):

break

cap.release()

cv2.destroyAllWindows()

drowsiness_installer.iss

#define MyAppName "Driver Drowsiness & Safety Monitoring"

#define MyAppVersion "1.0"

#define MyAppPublisher "Vishwa Tha Editor"

#define MyAppExeName "driver_drowsiness_gui.exe"

[Setup]

AppId={{5D4A9C4B-7E1B-4F2B-9C9B-1234567890AB}

AppName={#MyAppName}

AppVersion={#MyAppVersion}

AppPublisher={#MyAppPublisher}

DefaultDirName={pf}\DrowsinessMonitor

DefaultGroupName={#MyAppName}

DisableProgramGroupPage=no

OutputDir=release

OutputBaseFilename=DrowsinessMonitorSetup

Compression=lzma

SolidCompression=yes

WizardStyle=modern

[Languages]

Name: "english"; MessagesFile: "compiler:Default.isl"

[Files]

; EXE built by PyInstaller

Source: "release\driver_drowsiness_gui.exe"; DestDir: "{app}"; Flags: ignoreversion

[Icons]

; Start Menu shortcut

Name: "{group}\{#MyAppName}"; Filename: "{app}\{#MyAppExeName}"

; Desktop shortcut

Name: "{commondesktop}\{#MyAppName}"; Filename: "{app}\{#MyAppExeName}"

[Run]

; Run app automatically after install (optional)

Filename: "{app}\{#MyAppExeName}"; Description: "Launch {#MyAppName}"; Flags: nowait postinstall skipifsilent

Driver_drowsiness_gui.spec

# -*- mode: python ; coding: utf-8 -*-

a = Analysis(

['driver_drowsiness_gui.py'],

pathex=[],

binaries=[],

datas=[('shape_predictor_68_face_landmarks.dat', '.')],

hiddenimports=[],

hookspath=[],

hooksconfig={},

runtime_hooks=[],

excludes=[],

noarchive=False,

optimize=0,

)

pyz = PYZ(a.pure)

exe = EXE(

pyz,

a.scripts,

a.binaries,

a.datas,

[],

name='driver_drowsiness_gui',

debug=False,

bootloader_ignore_signals=False,

strip=False,

upx=True,

upx_exclude=[],

runtime_tmpdir=None,

console=False,

disable_windowed_traceback=False,

argv_emulation=False,

target_arch=None,

codesign_identity=None,

entitlements_file=None,

)

Driver_drowsiness_gui.py

import os

import sys

import cv2

import dlib

import numpy as np

from scipy.spatial import distance as dist

import time

import csv

from datetime import datetime

import platform

import threading

import tkinter as tk

from tkinter import ttk, messagebox

from PIL import Image, ImageTk  # pip install pillow

def resource_path(relative_path):

"""

Get absolute path to resource, works for dev and for PyInstaller .exe

"""

try:

base_path = sys._MEIPASS  # type: ignore[attr-defined]

except AttributeError:

base_path = os.path.dirname(os.path.abspath(__file__))

return os.path.join(base_path, relative_path)

# ========== CONFIG ==========

# Where to store logs & exported reports (user Documents -> DrowsinessMonitor)

LOG_DIR = os.path.join(os.path.expanduser("~"), "Documents", "DrowsinessMonitor")

LOG_FILENAME = os.path.join(LOG_DIR, "events_log.csv")

MODEL_PATH = resource_path("shape_predictor_68_face_landmarks.dat")

# How long (in frames) we allow NO FACE before auto-pausing

NO_FACE_CONSEC_FRAMES = 60  # ~2 sec at 30 FPS

USE_BEEP = True if platform.system() == "Windows" else False

if USE_BEEP:

import winsound

# ========== HELPER FUNCTIONS ==========

def eye_aspect_ratio(eye):

A = dist.euclidean(eye[1], eye[5])

B = dist.euclidean(eye[2], eye[4])

C = dist.euclidean(eye[0], eye[3])

ear = (A + B) / (2.0 * C)

return ear

def mouth_aspect_ratio(mouth):

# Outer mouth landmarks (48–60)

A = dist.euclidean(mouth[2], mouth[10])  # 51, 59

B = dist.euclidean(mouth[4], mouth[8])   # 53, 57

C = dist.euclidean(mouth[0], mouth[6])   # 49, 55

mar = (A + B) / (2.0 * C)

return mar

def shape_to_np(shape, dtype="int"):

coords = np.zeros((68, 2), dtype=dtype)

for i in range(0, 68):

coords[i] = (shape.part(i).x, shape.part(i).y)

return coords

def ensure_csv_header():

# Make sure log directory exists

os.makedirs(LOG_DIR, exist_ok=True)

try:

with open(LOG_FILENAME, mode="x", newline="") as f:

writer = csv.writer(f)

writer.writerow(["timestamp", "event", "ear", "mar"])

except FileExistsError:

pass

def log_event(event_type, ear_value, mar_value):

now = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

with open(LOG_FILENAME, mode="a", newline="") as f:

writer = csv.writer(f)

writer.writerow([now, event_type, f"{ear_value:.3f}", f"{mar_value:.3f}"])

def format_duration(td):

"""Return HH:MM:SS for a timedelta."""

total_sec = int(td.total_seconds())

hours = total_sec // 3600

minutes = (total_sec % 3600) // 60

seconds = total_sec % 60

return f"{hours:02d}:{minutes:02d}:{seconds:02d}"

# ========== SOUND MANAGER (REAL-TIME ALERTS) ==========

class SoundManager:

"""

Handles sound alerts.

- Continuous alarm for drowsiness (real-time)

- Short beep for yawns

"""

def __init__(self, is_enabled_callback, is_drowsy_callback):

self.is_enabled_callback = is_enabled_callback

self.is_drowsy_callback = is_drowsy_callback

self.yawn_playing = False

self.lock = threading.Lock()

# Start continuous drowsy alarm loop in background

t = threading.Thread(target=self._drowsy_loop, daemon=True)

t.start()

def _can_play(self):

return USE_BEEP and self.is_enabled_callback()

def _drowsy_loop(self):

"""

Continuous real-time loop:

If drowsy AND sound enabled -> keep beeping.

Stops automatically when drowsy flag becomes False.

"""

while True:

try:

if self._can_play() and self.is_drowsy_callback():

winsound.Beep(1600, 300)  # 300 ms beep

else:

time.sleep(0.05)

except Exception:

time.sleep(0.1)

def _play_pattern(self, pattern, flag_attr):

"""

pattern: list of (freq, duration_ms, pause_ms)

flag_attr: 'yawn_playing'

"""

try:

for freq, dur, pause in pattern:

if not self._can_play():

break

winsound.Beep(freq, dur)

if pause > 0:

time.sleep(pause / 1000.0)

except Exception:

pass

finally:

with self.lock:

setattr(self, flag_attr, False)

def trigger_yawn_alarm(self):

"""

Short tone for yawning (once per yawn event).

"""

if not self._can_play():

return

with self.lock:

if self.yawn_playing:

return

self.yawn_playing = True

pattern = [

(1200, 200, 40),

(900, 200, 0),

]

t = threading.Thread(target=self._play_pattern, args=(pattern, "yawn_playing"), daemon=True)

t.start()

def stop_all(self):

with self.lock:

self.yawn_playing = False

# drowsy loop auto-silences when is_drowsy_callback() becomes False

# ========== MAIN APP CLASS ==========

class DrowsinessApp:

def __init__(self, root):

self.root = root

self.root.title("Driver Drowsiness & Safety Monitoring")

self.root.geometry("1000x620")

self.root.resizable(False, False)

# --- State variables ---

self.cap = None

self.running = False

self.processing_thread = None

# Auto-pause (no driver)

self.no_face_counter = 0

self.no_driver_paused = False

# Session timing

self.session_start_time = None

self.session_end_time = None

self.first_event_time = None

self.last_event_time = None

# Shared data

self.frame_lock = threading.Lock()

self.latest_frame = None

self.ear = 0.0

self.mar = 0.0

self.eye_counter = 0

self.mouth_counter = 0

self.drowsy_events = 0

self.yawn_events = 0

self.drowsy_alarm_on = False   # used by real-time sound

self.yawn_alarm_on = False

self.current_status_text = "IDLE"

self.current_status_color = "gray"

# Sensitivity profiles (yawn more sensitive)

self.sensitivity_profiles = {

"High":   {"ear_thresh": 0.25, "ear_frames": 15, "mar_thresh": 0.40, "mar_frames": 6},

"Normal": {"ear_thresh": 0.23, "ear_frames": 20, "mar_thresh": 0.45, "mar_frames": 8},

"Low":    {"ear_thresh": 0.21, "ear_frames": 25, "mar_thresh": 0.50, "mar_frames": 10},

}

self.current_profile_name = tk.StringVar(value="Normal")

self.eye_ar_thresh = self.sensitivity_profiles["Normal"]["ear_thresh"]

self.eye_ar_frames = self.sensitivity_profiles["Normal"]["ear_frames"]

self.mouth_ar_thresh = self.sensitivity_profiles["Normal"]["mar_thresh"]

self.mouth_ar_frames = self.sensitivity_profiles["Normal"]["mar_frames"]

self.beep_enabled = tk.BooleanVar(value=True)

# Vehicle dashboard

self.speed = tk.IntVar(value=60)   # km/h

self.safety_score = 100            # 0–100

# Dlib models

try:

self.detector = dlib.get_frontal_face_detector()

self.predictor = dlib.shape_predictor(MODEL_PATH)

except Exception as e:

messagebox.showerror("Error", f"Error loading landmark model:\n{e}")

raise

# Landmark indices

self.RIGHT_EYE_START, self.RIGHT_EYE_END = 36, 42

self.LEFT_EYE_START, self.LEFT_EYE_END = 42, 48

self.MOUTH_START, self.MOUTH_END = 48, 61

ensure_csv_header()

# Sound manager (real-time alerts)

self.sound_manager = SoundManager(self._is_beep_enabled, self._is_drowsy_active)

self._build_ui()

def _is_beep_enabled(self):

return self.beep_enabled.get()

def _is_drowsy_active(self):

return self.drowsy_alarm_on

# ========== UI BUILD ==========

def _build_ui(self):

# Video

video_frame = ttk.LabelFrame(self.root, text="Live View")

video_frame.place(x=10, y=10, width=640, height=480)

self.video_label = ttk.Label(video_frame)

self.video_label.place(x=0, y=0, width=640, height=450)

# Right panel

ctrl_frame = ttk.LabelFrame(self.root, text="Controls & Status")

ctrl_frame.place(x=660, y=10, width=320, height=580)

# Start / Stop

self.start_btn = ttk.Button(ctrl_frame, text="Start Monitoring", command=self.start_monitoring)

self.start_btn.pack(pady=(10, 5), fill="x", padx=10)

self.stop_btn = ttk.Button(

ctrl_frame,

text="Stop Monitoring",

command=self.stop_monitoring,

state="disabled"

)

self.stop_btn.pack(pady=5, fill="x", padx=10)

# Sensitivity

ttk.Label(ctrl_frame, text="Sensitivity").pack(pady=(10, 5), anchor="w", padx=10)

self.sens_combo = ttk.Combobox(

ctrl_frame,

textvariable=self.current_profile_name,

values=list(self.sensitivity_profiles.keys()),

state="readonly"

)

self.sens_combo.pack(pady=5, fill="x", padx=10)

self.sens_combo.bind("<<ComboboxSelected>>", self.on_sensitivity_change)

# Beep checkbox

self.beep_check = ttk.Checkbutton(

ctrl_frame,

text="Enable Sound Alerts",

variable=self.beep_enabled

)

self.beep_check.pack(pady=5, anchor="w", padx=10)

ttk.Separator(ctrl_frame, orient="horizontal").pack(fill="x", pady=8)

# Status

ttk.Label(ctrl_frame, text="System Status:").pack(anchor="w", padx=10)

self.status_label = ttk.Label(ctrl_frame, text="IDLE", foreground="gray")

self.status_label.pack(anchor="w", padx=20, pady=(0, 8))

ttk.Label(ctrl_frame, text="EAR (Eye Aspect Ratio):").pack(anchor="w", padx=10)

self.ear_label = ttk.Label(ctrl_frame, text="0.00")

self.ear_label.pack(anchor="w", padx=20)

ttk.Label(ctrl_frame, text="MAR (Mouth Aspect Ratio):").pack(anchor="w", padx=10, pady=(8, 0))

self.mar_label = ttk.Label(ctrl_frame, text="0.00")

self.mar_label.pack(anchor="w", padx=20)

ttk.Separator(ctrl_frame, orient="horizontal").pack(fill="x", pady=8)

# Events

ttk.Label(ctrl_frame, text="Drowsy Events:").pack(anchor="w", padx=10)

self.drowsy_count_label = ttk.Label(ctrl_frame, text="0")

self.drowsy_count_label.pack(anchor="w", padx=20)

ttk.Label(ctrl_frame, text="Yawn Events:").pack(anchor="w", padx=10, pady=(8, 0))

self.yawn_count_label = ttk.Label(ctrl_frame, text="0")

self.yawn_count_label.pack(anchor="w", padx=20)

ttk.Separator(ctrl_frame, orient="horizontal").pack(fill="x", pady=8)

# Speed + Safety

ttk.Label(ctrl_frame, text="Vehicle Speed (km/h):").pack(anchor="w", padx=10)

self.speed_value_label = ttk.Label(ctrl_frame, text=f"{self.speed.get()} km/h")

self.speed_value_label.pack(anchor="w", padx=20)

self.speed_scale = ttk.Scale(

ctrl_frame,

from_=0,

to=140,

orient="horizontal",

variable=self.speed,

command=self.on_speed_change

)

self.speed_scale.pack(fill="x", padx=10, pady=(5, 5))

ttk.Label(ctrl_frame, text="Safety Score:").pack(anchor="w", padx=10, pady=(8, 0))

self.safety_score_label = ttk.Label(ctrl_frame, text="100 / 100")

self.safety_score_label.pack(anchor="w", padx=20)

self.safety_canvas = tk.Canvas(

ctrl_frame,

width=270,

height=22,

bg="#222222",

highlightthickness=0

)

self.safety_canvas.pack(padx=10, pady=(5, 5))

ttk.Separator(ctrl_frame, orient="horizontal").pack(fill="x", pady=8)

# Data export

ttk.Label(ctrl_frame, text="Data Export (for Cloud / Report):").pack(anchor="w", padx=10)

export_frame1 = ttk.Frame(ctrl_frame)

export_frame1.pack(fill="x", padx=10, pady=(5, 2))

self.export_csv_btn = ttk.Button(

export_frame1,

text="Export CSV",

command=self.export_session_csv

)

self.export_csv_btn.pack(side="left", expand=True, fill="x")

self.export_json_btn = ttk.Button(

export_frame1,

text="Export JSON",

command=self.export_session_json

)

self.export_json_btn.pack(side="left", expand=True, fill="x", padx=(5, 0))

export_frame2 = ttk.Frame(ctrl_frame)

export_frame2.pack(fill="x", padx=10, pady=(2, 5))

self.export_html_btn = ttk.Button(

export_frame2,

text="Export HTML Report",

command=self.export_session_html

)

self.export_html_btn.pack(side="left", expand=True, fill="x")

# Footer

footer = ttk.Label(

self.root,

text="Start monitoring → real-time alerts → export CSV/JSON/HTML (saved in Documents\\DrowsinessMonitor).",

anchor="center"

)

footer.place(x=10, y=580, width=980, height=30)

self.root.protocol("WM_DELETE_WINDOW", self.on_close)

# ========== UI ACTIONS ==========

def on_sensitivity_change(self, event=None):

profile_name = self.current_profile_name.get()

profile = self.sensitivity_profiles.get(

profile_name,

self.sensitivity_profiles["Normal"]

)

self.eye_ar_thresh = profile["ear_thresh"]

self.eye_ar_frames = profile["ear_frames"]

self.mouth_ar_thresh = profile["mar_thresh"]

self.mouth_ar_frames = profile["mar_frames"]

def on_speed_change(self, value):

self.speed_value_label.config(text=f"{self.speed.get()} km/h")

def start_monitoring(self):

if self.running:

return

self.cap = cv2.VideoCapture(0)

if not self.cap.isOpened():

messagebox.showerror("Error", "Could not open webcam.")

return

self.running = True

self.start_btn.config(state="disabled")

self.stop_btn.config(state="normal")

# reset counters and times

self.eye_counter = 0

self.mouth_counter = 0

self.drowsy_alarm_on = False

self.yawn_alarm_on = False

self.drowsy_events = 0

self.yawn_events = 0

self.first_event_time = None

our_last = None

self.last_event_time = None

self.session_start_time = datetime.now()

self.session_end_time = None

self.no_face_counter = 0

self.no_driver_paused = False

self.current_status_text = "RUNNING"

self.current_status_color = "green"

# Start background processing thread

self.processing_thread = threading.Thread(

target=self.process_loop,

daemon=True

)

self.processing_thread.start()

# Start GUI update loop

self.update_gui()

def stop_monitoring(self):

if not self.running:

return

self.session_end_time = datetime.now()

self.running = False

if self.processing_thread is not None:

self.processing_thread = None

if self.cap is not None and self.cap.isOpened():

self.cap.release()

self.cap = None

with self.frame_lock:

self.latest_frame = None

self.drowsy_alarm_on = False

self.yawn_alarm_on = False

self.sound_manager.stop_all()

self.status_label.config(text="STOPPED", foreground="orange")

self.start_btn.config(state="normal")

self.stop_btn.config(state="disabled")

self.video_label.config(image="")

self._show_session_summary()

def _show_session_summary(self):

"""

Show session summary in a custom popup window

with Export CSV / JSON / HTML buttons.

"""

if self.session_start_time is None or self.session_end_time is None:

return

# build summary text

duration = self.session_end_time - self.session_start_time

duration_str = format_duration(duration)

total_events = self.drowsy_events + self.yawn_events

hours = duration.total_seconds() / 3600.0 if duration.total_seconds() > 0 else 0

if hours > 0:

rate = total_events / hours

rate_str = f"{rate:.2f} events/hour"

else:

rate_str = "N/A"

if self.first_event_time is not None:

first_str = self.first_event_time.strftime("%Y-%m-%d %H:%M:%S")

last_str = self.last_event_time.strftime("%Y-%m-%d %H:%M:%S")

else:

first_str = "No events"

last_str = "No events"

summary_text = (

"Session Summary\n\n"

f"Monitoring duration : {duration_str}\n"

f"Total events        : {total_events}\n"

f"  - Drowsy events   : {self.drowsy_events}\n"

f"  - Yawn events     : {self.yawn_events}\n"

f"Event rate          : {rate_str}\n\n"

f"First event time    : {first_str}\n"

f"Last event time     : {last_str}\n\n"

f"Logs & reports saved in:\n{LOG_DIR}"

)

# custom popup window

win = tk.Toplevel(self.root)

win.title("Session Summary")

win.resizable(False, False)

win.grab_set()

lbl = tk.Label(

win,

text=summary_text,

justify="left",

font=("Segoe UI", 9)

)

lbl.pack(padx=15, pady=(15, 10))

# Export buttons

btn_frame = ttk.Frame(win)

btn_frame.pack(padx=15, pady=(0, 15), fill="x")

csv_btn = ttk.Button(

btn_frame,

text="Export CSV",

command=self.export_session_csv

)

csv_btn.pack(side="left", expand=True, fill="x")

json_btn = ttk.Button(

btn_frame,

text="Export JSON",

command=self.export_session_json

)

json_btn.pack(side="left", expand=True, fill="x", padx=5)

html_btn = ttk.Button(

btn_frame,

text="Export HTML Report",

command=self.export_session_html

)

html_btn.pack(side="left", expand=True, fill="x")

# Close button row

close_frame = ttk.Frame(win)

close_frame.pack(padx=15, pady=(0, 10), fill="x")

close_btn = ttk.Button(close_frame, text="Close", command=win.destroy)

close_btn.pack(side="right")

def on_close(self):

if self.running:

self.stop_monitoring()

self.root.destroy()

# ========== CORE PROCESSING (BACKGROUND THREAD) ==========

def _record_event_time(self):

now = datetime.now()

if self.first_event_time is None:

self.first_event_time = now

self.last_event_time = now

def process_loop(self):

while self.running and self.cap is not None and self.cap.isOpened():

ret, frame = self.cap.read()

if not ret:

break

frame = cv2.resize(frame, (640, 480))

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

rects = self.detector(gray, 0)

ear = 0.0

mar = 0.0

drowsy_detected = False

yawn_detected = False

if len(rects) > 0:

# Reset "no face" counter and unpause if needed

self.no_face_counter = 0

if self.no_driver_paused:

self.no_driver_paused = False

rect = rects[0]

shape = self.predictor(gray, rect)

shape = shape_to_np(shape)

# Eyes

leftEye = shape[self.LEFT_EYE_START:self.LEFT_EYE_END]

rightEye = shape[self.RIGHT_EYE_START:self.RIGHT_EYE_END]

leftEAR = eye_aspect_ratio(leftEye)

rightEAR = eye_aspect_ratio(rightEye)

ear = (leftEAR + rightEAR) / 2.0

leftEyeHull = cv2.convexHull(leftEye)

rightEyeHull = cv2.convexHull(rightEye)

cv2.drawContours(frame, [leftEyeHull], -1, (0, 255, 0), 1)

cv2.drawContours(frame, [rightEyeHull], -1, (0, 255, 0), 1)

# Mouth

mouth = shape[self.MOUTH_START:self.MOUTH_END]

mar = mouth_aspect_ratio(mouth)

mouthHull = cv2.convexHull(mouth)

cv2.drawContours(frame, [mouthHull], -1, (255, 0, 0), 1)

# Only process drowsiness/yawn if NOT paused

if not self.no_driver_paused:

# Drowsiness (eyes)

if ear < self.eye_ar_thresh:

self.eye_counter += 1

else:

self.eye_counter = 0

self.drowsy_alarm_on = False  # turn off alarm when eyes open

if self.eye_counter >= self.eye_ar_frames:

drowsy_detected = True

self.drowsy_alarm_on = True

if self.eye_counter == self.eye_ar_frames:

self.drowsy_events += 1

self._record_event_time()

log_event("DROWSY", ear, mar)

# Yawning (mouth)

if mar > self.mouth_ar_thresh:

self.mouth_counter += 1

else:

self.mouth_counter = 0

self.yawn_alarm_on = False

if self.mouth_counter >= self.mouth_ar_frames:

yawn_detected = True

if not self.yawn_alarm_on:

self.yawn_alarm_on = True

self.yawn_events += 1

self._record_event_time()

log_event("YAWN", ear, mar)

self.sound_manager.trigger_yawn_alarm()

# Text overlays

if drowsy_detected:

cv2.putText(

frame,

"DROWSY!",

(10, 60),

cv2.FONT_HERSHEY_SIMPLEX,

1.0,

(0, 0, 255),

3

)

if yawn_detected:

cv2.putText(

frame,

"YAWNING!",

(10, 100),

cv2.FONT_HERSHEY_SIMPLEX,

1.0,

(255, 0, 255),

3

)

cv2.putText(

frame,

f"EAR: {ear:.2f}",

(10, 20),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255, 255, 0),

2

)

cv2.putText(

frame,

f"MAR: {mar:.2f}",

(10, 40),

cv2.FONT_HERSHEY_SIMPLEX,

0.6,

(255, 255, 0),

2

)

# Status

if self.no_driver_paused:

self.current_status_text = "PAUSED (NO DRIVER)"

self.current_status_color = "orange"

else:

if drowsy_detected:

self.current_status_text = "DROWSY"

self.current_status_color = "red"

elif yawn_detected:

self.current_status_text = "YAWNING"

self.current_status_color = "purple"

else:

self.current_status_text = "RUNNING"

self.current_status_color = "green"

else:

# No face detected

self.no_face_counter += 1

self.drowsy_alarm_on = False  # no driver -> no drowsy alarm sound

if self.no_face_counter >= NO_FACE_CONSEC_FRAMES:

self.no_driver_paused = True

self.current_status_text = "PAUSED (NO DRIVER)"

self.current_status_color = "orange"

else:

self.current_status_text = "NO DRIVER"

self.current_status_color = "red"

cv2.putText(

frame,

"NO FACE DETECTED",

(10, 30),

cv2.FONT_HERSHEY_SIMPLEX,

0.7,

(0, 0, 255),

2

)

# Update shared metrics

self.ear = ear

self.mar = mar

with self.frame_lock:

self.latest_frame = frame.copy()

time.sleep(0.005)

self.running = False

self.drowsy_alarm_on = False

self.yawn_alarm_on = False

# ========== SAFETY SCORE & GAUGE ==========

def _calculate_safety_score(self):

if self.no_driver_paused:

self.safety_score = 100

return

score = 100

if self.current_status_text == "DROWSY":

score -= 40

elif self.current_status_text == "YAWNING":

score -= 20

total_events = self.drowsy_events + self.yawn_events

score -= min(total_events * 3, 30)

v = self.speed.get()

if v > 100:

score -= 25

elif v > 80:

score -= 15

elif v > 60:

score -= 8

score = max(0, min(100, score))

self.safety_score = score

def _update_safety_display(self):

self._calculate_safety_score()

s = self.safety_score

self.safety_score_label.config(text=f"{int(s)} / 100")

self.safety_canvas.delete("all")

width = 270

height = 22

margin = 2

fill_width = int((width - 2 * margin) * (s / 100.0))

if s >= 70:

color = "#00c853"  # green

elif s >= 40:

color = "#ffab00"  # yellow

else:

color = "#d50000"  # red

self.safety_canvas.create_rectangle(

margin, margin, width - margin, height - margin,

outline="#444444", fill="#111111"

)

if fill_width > 0:

self.safety_canvas.create_rectangle(

margin, margin,

margin + fill_width, height - margin,

outline="", fill=color

)

self.safety_canvas.create_rectangle(

margin, margin, width - margin, height - margin,

outline="#ffffff"

)

# ========== DATA EXPORT HELPERS ==========

def _get_session_events(self):

if self.session_start_time is None or self.session_end_time is None:

return []

rows = []

try:

with open(LOG_FILENAME, newline="") as f:

reader = csv.DictReader(f)

for row in reader:

try:

ts = datetime.strptime(row["timestamp"], "%Y-%m-%d %H:%M:%S")

except Exception:

continue

if self.session_start_time <= ts <= self.session_end_time:

rows.append({

"timestamp": row["timestamp"],

"event": row["event"],

"ear": row["ear"],

"mar": row["mar"],

})

except FileNotFoundError:

pass

return rows

def export_session_csv(self):

if self.session_start_time is None or self.session_end_time is None:

messagebox.showwarning(

"Export CSV",

"No completed session. Start and stop monitoring first."

)

return

os.makedirs(LOG_DIR, exist_ok=True)

session_id = self.session_start_time.strftime("session_%Y%m%d_%H%M%S")

filename = os.path.join(LOG_DIR, f"{session_id}.csv")

rows = self._get_session_events()

if not rows:

messagebox.showinfo(

"Export CSV",

"No events recorded in this session. CSV not created."

)

return

with open(filename, mode="w", newline="") as f:

writer = csv.writer(f)

writer.writerow(["timestamp", "event", "ear", "mar"])

for r in rows:

writer.writerow([r["timestamp"], r["event"], r["ear"], r["mar"]])

messagebox.showinfo(

"Export CSV",

f"Session data exported to:\n{filename}"

)

def export_session_json(self):

import json

if self.session_start_time is None or self.session_end_time is None:

messagebox.showwarning(

"Export JSON",

"No completed session. Start and stop monitoring first."

)

return

os.makedirs(LOG_DIR, exist_ok=True)

session_id = self.session_start_time.strftime("session_%Y%m%d_%H%M%S")

filename = os.path.join(LOG_DIR, f"{session_id}.json")

rows = self._get_session_events()

summary = {

"session_start": self.session_start_time.strftime("%Y-%m-%d %H:%M:%S")

if self.session_start_time else None,

"session_end": self.session_end_time.strftime("%Y-%m-%d %H:%M:%S")

if self.session_end_time else None,

"drowsy_events": self.drowsy_events,

"yawn_events": self.yawn_events,

"total_events": self.drowsy_events + self.yawn_events,

}

data = {

"summary": summary,

"events": rows,

}

with open(filename, "w", encoding="utf-8") as f:

json.dump(data, f, indent=2)

messagebox.showinfo(

"Export JSON",

f"Session JSON exported to:\n{filename}"

)

def export_session_html(self):

"""

Generate a simple HTML report for the last completed session.

Includes summary + event table.

"""

if self.session_start_time is None or self.session_end_time is None:

messagebox.showwarning(

"Export HTML",

"No completed session. Start and stop monitoring first."

)

return

os.makedirs(LOG_DIR, exist_ok=True)

session_id = self.session_start_time.strftime("session_%Y%m%d_%H%M%S")

filename = os.path.join(LOG_DIR, f"{session_id}.html")

rows = self._get_session_events()

duration_str = "N/A"

if self.session_end_time and self.session_start_time:

duration = self.session_end_time - self.session_start_time

duration_str = format_duration(duration)

total_events = self.drowsy_events + self.yawn_events

first_str = (

self.first_event_time.strftime("%Y-%m-%d %H:%M:%S")

if self.first_event_time else "No events"

)

last_str = (

self.last_event_time.strftime("%Y-%m-%d %H:%M:%S")

if self.last_event_time else "No events"

)

html = f"""<!DOCTYPE html>

<html>

<head>

<meta charset="utf-8">

<title>Driver Drowsiness Session Report - {session_id}</title>

<style>

body {{

font-family: Arial, sans-serif;

background-color: #111827;

color: #e5e7eb;

margin: 0;

padding: 20px;

}}

h1 {{

color: #38bdf8;

}}

h2 {{

color: #a5b4fc;

}}

.section {{

margin-bottom: 24px;

padding: 16px;

background-color: #1f2937;

border-radius: 8px;

border: 1px solid #374151;

}}

.summary-table td {{

padding: 4px 12px;

}}

table.events {{

width: 100%;

border-collapse: collapse;

margin-top: 8px;

}}

table.events th, table.events td {{

border: 1px solid #4b5563;

padding: 6px 8px;

font-size: 14px;

}}

table.events th {{

background-color: #111827;

}}

tr.event-drowsy td {{

background-color: #7f1d1d;

}}

tr.event-yawn td {{

background-color: #312e81;

}}

.footer {{

margin-top: 24px;

font-size: 12px;

color: #9ca3af;

text-align: center;

}}

</style>

</head>

<body>

<h1>Driver Drowsiness & Safety Monitoring - Session Report</h1>

<div class="section">

<h2>Session Summary</h2>

<table class="summary-table">

<tr><td><b>Session ID</b></td><td>{session_id}</td></tr>

<tr><td><b>Start Time</b></td><td>{self.session_start_time.strftime("%Y-%m-%d %H:%M:%S")}</td></tr>

<tr><td><b>End Time</b></td><td>{self.session_end_time.strftime("%Y-%m-%d %H:%M:%S")}</td></tr>

<tr><td><b>Duration</b></td><td>{duration_str}</td></tr>

<tr><td><b>Total Events</b></td><td>{total_events}</td></tr>

<tr><td>&bull; Drowsy Events</td><td>{self.drowsy_events}</td></tr>

<tr><td>&bull; Yawn Events</td><td>{self.yawn_events}</td></tr>

<tr><td><b>First Event</b></td><td>{first_str}</td></tr>

<tr><td><b>Last Event</b></td><td>{last_str}</td></tr>

</table>

</div>

<div class="section">

<h2>Event Timeline</h2>

"""

if rows:

html += """

<table class="events">

<thead>

<tr>

<th>#</th>

<th>Timestamp</th>

<th>Event</th>

<th>EAR</th>

<th>MAR</th>

</tr>

</thead>

<tbody>

"""

for idx, r in enumerate(rows, start=1):

ev = r["event"].upper()

row_class = ""

if ev == "DROWSY":

row_class = "event-drowsy"

elif ev == "YAWN":

row_class = "event-yawn"

html += f"""      <tr class="{row_class}">

<td>{idx}</td>

<td>{r["timestamp"]}</td>

<td>{ev}</td>

<td>{r["ear"]}</td>

<td>{r["mar"]}</td>

</tr>

"""

html += """    </tbody>

</table>

"""

else:

html += "<p>No events recorded during this session.</p>\n"

html += f"""

</div>

<div class="footer">

Generated by Driver Drowsiness & Safety Monitoring System.<br>

Logs & reports saved in: {LOG_DIR}

</div>

</body>

</html>

"""

try:

with open(filename, "w", encoding="utf-8") as f:

f.write(html)

messagebox.showinfo(

"Export HTML",

f"HTML report exported to:\n{filename}"

)

except Exception as e:

messagebox.showerror("Export HTML", f"Failed to write HTML file:\n{e}")

# ========== GUI UPDATE LOOP ==========

def update_gui(self):

if not self.running:

return

frame_to_show = None

with self.frame_lock:

if self.latest_frame is not None:

frame_to_show = self.latest_frame.copy()

if frame_to_show is not None:

frame_rgb = cv2.cvtColor(frame_to_show, cv2.COLOR_BGR2RGB)

img = Image.fromarray(frame_rgb)

imgtk = ImageTk.PhotoImage(image=img)

self.video_label.imgtk = imgtk

self.video_label.configure(image=imgtk)

self.ear_label.config(text=f"{self.ear:.2f}")

self.mar_label.config(text=f"{self.mar:.2f}")

self.status_label.config(text=self.current_status_text, foreground=self.current_status_color)

self.drowsy_count_label.config(text=str(self.drowsy_events))

self.yawn_count_label.config(text=str(self.yawn_events))

self.speed_value_label.config(text=f"{self.speed.get()} km/h")

self._update_safety_display()

self.root.after(30, self.update_gui)

# ========== RUN APP ==========

if __name__ == "__main__":

root = tk.Tk()

style = ttk.Style(root)

try:

style.theme_use("clam")

except Exception:

pass

app = DrowsinessApp(root)

root.mainloop()